-----OFFICIAL INFORMATION DISPATCH FOLLOWS-----

RTTUZYUW RUCBTFA0039 2421633-UUUU--RHMCSUU.

ZNR UUUUU

R 291633Z AUG 12

FM COMNAVSURFLANT NORFOLK VA

TO ALNAVSURFLANT

INFO COMUSFLTFORCOM NORFOLK VA

COMNAVSURFPAC SAN DIEGO CA

COMNAVSURFLANT NORFOLK VA

BT

UNCLAS

SECINFO/U/-//

MSGID/GENADMIN,USMTF,2008/COMNAVSURFLANT NORFOLK VA/-/AUG//

SUBJ/COMNAVSURFLANT REGULATIONS - SERIAL 005/TOPSIDE APPEARANCE//

REF/A/MSGID:P4/COMNAVSURFLANT/292020ZJUN2012//

REF/B/MSGID:RMG/COMNAVSURFLANT/311554ZJUL2012//

REF/C/DESC:INST/NSTM 631/01NOV2008//

REF/D/MSGID:RMG/COMNAVSURFLANT/131818ZJUL2012//

REF/E/MSGID:RMG/COMNAVSURFLANT/091535ZMAR2012//

REF/F/DESC:DOC/MIP 6121/JAN2012//

REF/G/DESC:INST/NSTM 613/01JUL2008//

REF/H/DESC:INST/NSTM 600 V3/01OCT2012//

REF/I/DESC:INST/CNRMA/03APR2006//

REF/J/DESC:INST/NASTA MYPT/11JUL2006//

NARR/REF A IS THE CNSL REGULATIONS ANNOUNCEMENT MESSAGE.  REF B IS

CNSL REGULATIONS SERIAL 002 - SHIP'S ROUTINE.  REF C IS THE NAVAL

SHIP'S TECHNICAL MANUAL REGARDING PRESERVATION OF SHIPS IN SERVICE.

REF D IS THE CNSL MESSAGE DIRECTING FRESHWATER WASHDOWN

REQUIREMENTS.  REF E IS THE CNSL MESSAGE DIRECTING THE USE OF PEEL

AND STICK NON-SKID.  REF F IS MIP/MRC SERIES 6121/S-12, CLEAN AND

INSPECT NYLON OR POLYESTER GUARD LINES.  REF G IS THE NAVAL SHIP'S

TECHNICAL MANUAL REGARDING WIRE AND FIBER ROPE AND RIGGING.  REF H

IS THE NAVAL SHIP'S TECHNICAL MANUAL REGARDING HULL OUTFITTING

EQUIPMENT.  REF I IS THE COMMANDER NAVY REGION MID-ATLANTIC SOPA

(ADMIN) INSTRUCTION 5400.1A.  REF J IS NAVAL STATION MAYPORT SOPA

INSTRUCTION 5440.3G.//

POC/HAYHURST, J/CAPT/UNIT:CNSL N436/NAME:NORVA/TEL:757-445-5684

/TEL:DSN 836-5684/EMAIL:JEFFREY.K.HAYHURST@NAVY.MIL//

GENTEXT/REMARKS/

1.  THE PURPOSE OF THIS MESSAGE IS TO DIRECT TOPSIDE APPEARANCE

REQUIREMENTS.  THIS MESSAGE IS SERIAL 005 OF COMNAVSURFLANT

REGULATIONS ANNOUNCED IN REF A.

2.  BACKGROUND.  A SMART TOPSIDE APPEARANCE IS THE DIRECT RESULT OF

A SHIP THAT MAINTAINS COMPLIANCE WITH REQUIRED STANDARDS OF SAFETY,

MATERIAL READINESS, AND CLEANLINESS.  A CLEAN, DRY, AND

WELL-PRESERVED TOPSIDE REDUCES THE MAN-HOURS AND COSTS ASSOCIATED

WITH CORROSION REPAIRS.

3.  ACTION.  TO ENSURE THE MILITARY SMARTNESS, PROPER STOWAGE, AND

CLEANLINESS OF TOPSIDE AREAS, SHIPS SHALL COMPLY WITH THE FOLLOWING

DIRECTION:

A.  SWEEPERS.  CONDUCT DAILY TOPSIDE SWEEPERS PER REF B.

B.  PAINT.  MAINTAIN TOPSIDE AREAS PER REF C.  DO NOT PAINT SOLELY

FOR COSMETIC REASONS; PAINTING IS THE LAST STEP OF A THOROUGH

SURFACE PREPARATION PROCESS TO PREVENT CORROSION.  DO NOT PAINT OVER

RUNNING RUST.  INSTEAD, PERFORM REGULAR FRESHWATER WASHDOWNS AND

SCRUB AWAY RUNNING RUST.

C.  FRESHWATER WASHDOWNS.  CONDUCT FRESHWATER WASHDOWNS PER REF D TO

REDUCE THE ACCELERANTS OF CORROSION.

D.  SIDES.  PAY PARTICULAR ATTENTION TO TOPSIDE AREAS IVO DISCHARGES

AND EXHAUSTS.  MAINTAIN BOW NUMBERS, SHIP'S NAME, AND DRAFT MARKINGS

PER REF C.

E.  NON-SKID.  REGULARLY INSPECT NON-SKID DURING DAILY SWEEPERS FOR

SIGNS OF WEAR AND CORROSION.  DOCUMENT DISCREPANCIES IN THE CSMP AND

FOLLOW UP UNTIL CORRECTED.  USE PEEL AND STICK NON-SKID IN

NON-CRITICAL AREAS PER REF E.

F.  LIFELINES.  LIFELINES SHALL PRESENT A SMART MILITARY APPEARANCE,

BE IN GOOD MATERIAL CONDITION, CLEANED, PROPERLY ADJUSTED, AND

INSPECTED PER REF F.  ENSURE LIFELINES ARE PROPERLY ATTACHED TO

STANCHIONS ON BOTH ENDS WITH APPROVED HARDWARE (E.G., SHACKLES, SNAP

HOOKS).  ENSURE THE HARDWARE IS FULLY OPERABLE AND FREE OF

CORROSION.  LIFELINES SHALL NOT BE SLEEVED, COVERED WITH CANVAS,

FITTED WITH TURKSHEAD, OR PAINTED.  CHAFING MATERIAL SHALL BE FITTED

AND UTILIZED ON LIFELINE POINTS OF CONTACT WITH STANCHIONS.

TEMPORARY LIFELINES (E.G., AROUND THE BROW) SHALL MEET APPLICABLE

SAFETY REQUIREMENTS AND RIGGED AS TO DISPLAY A NEAT APPEARANCE.

G.  MOORING LINES.  ENSURE MOORING LINES ARE NOT FRAYED OR SOILED

AND ARE SERVICEABLE IAW REF G.  THEY SHALL BE PROPERLY BIRD-NESTED

OR, IF GETTING UNDERWAY, FAKED-OUT ON DECK.  RAT GUARDS AND CHAFFING

GEAR SHALL BE PROPERLY INSTALLED AT THE PIER AND SHIP.  MOORING

LINES SHALL BE PROPERLY FRAPPED AND TATTLETALE INSTALLED, WHERE

APPLICABLE.

H.  ADDITIONAL LINES.  ADDITIONAL LINES MADE UP TO THE SHIP (E.G.,

FENDERS, BARGES, ETC.) SHALL BE SERVICEABLE IAW REF G AND PROPERLY

SECURED TO APPROPRIATE DECK FITTINGS WHILE ENSURING EXTRA LINE IS

BIRD-NESTED OR FLEMISHED ON DECK.

I.  TOPSIDE CLEANING GEAR.  STOW ALL TOPSIDE CLEANING GEAR (E.G.,

SWABS, BROOMS, BUCKETS, DETERGENT BOTTLES, SCOURING PADS, ETC.) IN

APPROVED GEAR LOCKERS OR STORAGE AREAS.

J.  SMOKING AREA.  CLEAN DESIGNATED SMOKING AREAS DAILY.

K.  TOMPIONS.  TOMPIONS SHALL BE INSTALLED WHILE IN PORT AND EXPOSED

METAL SURFACES SHALL BE HIGHLY POLISHED.  TOMPIONS SHALL BE REMOVED

WHILE AT SEA AND MUZZLE BAGS INSTALLED AS APPLICABLE.

L.  BRIGHTWORK.  BRIGHTWORK, LABEL PLATES, METAL WATERTIGHT CLOSURE

LABELS, AND DIRECTIONAL SIGNS SHALL BE KEPT FREE OF PAINT,

VERDIGRIS, AND RUST, AND SHINED REGULARLY.

M.  BELL.  THE SHIP'S BELL SHALL BE MAINTAINED IN WORKING ORDER AND

POLISHED.

N.  ANCHORS, CHAINS, AND DECK EQUIPMENT.  TURTLEBACKS SHALL BE

PROPERLY INSTALLED AND SECURELY FASTENED.  ANCHOR CHAINS SHALL BE

PRESERVED AND PAINTED PER REF C.  CAPSTANS SHALL BE FREE OF

CORROSION AND PROPERLY PRESERVED.  IF THE CAPSTAN TOP IS BRASS, IT

SHALL BE POLISHED.

O.  LADDERS.  TOPSIDE LADDERS SHALL BE CLEAN AND PRESENT A SMART

MILITARY APPEARANCE.  ENSURE LADDERS ARE IN GOOD MATERIAL CONDITION

AND DECK TREADS, HANDRAILS, AND ATTACHED HARDWARE ARE PROPERLY

ATTACHED IAW REF H.

P.  OVERBOARD DISCHARGES.  ENSURE OVERBOARD DISCHARGES DO NOT SPRAY

DIRECTLY ONTO PIERS, FLOATS, OR BARGES TIED ALONGSIDE.

Q.  RUBBER SCUPPERS.  WHERE APPLICABLE, RUBBER SCRUBBERS SHALL BE

UTILIZED, FREE OF PAINT, AND PROPERLY FASTENED.

R.  EQUIPMENT ALIGNMENT.  MAINTAIN ALL ROTATIONAL EQUIPMENT (E.G.,

WEAPONS, RADAR SYSTEMS) IN REQUIRED STOWAGE/SECURED POSITION

(CENTERLINED WHERE APPLICABLE).

S.  QUARTERDECK.  THE QUARTERDECK AND ASSIGNED WATCHSTANDERS SHALL

PRESENT A CLEAN, PROFESSIONAL MILITARY APPEARANCE AT ALL TIMES.

DISPLAY THE APPROPRIATE NAME BOARDS, BULLETS, AND SHIPS MATTING.

ENSURE THE PODIUM IS IN GOOD MATERIAL CONDITION, FREE OF NICKS OR

DAMAGE, AND, IF OF WOOD CONSTRUCTION, PROPERLY PRESERVED WITH THE

APPROPRIATE STAIN OR LACQUER.

T.  SQUADRON SIGN OR BADGE.  SQUADRON SIGNS OR BADGES SHALL BE

MOUNTED, AS APPLICABLE, AND PRESENT A BRIGHT, WELL-PAINTED

APPEARANCE.

U.  BROWS.  THE SHIPS BROW SHALL BE CLEAN, PROPERLY SECURED, AND

PRESENT A SMART MILITARY APPEARANCE.  LINES AND TIE-DOWN CHAINS

SHALL BE CLEAN AND WELL PRESERVED.

V.  BROW SKIRTS.  BROW SKIRTS SHALL BE SECURELY FASTENED TO THE BROW

AND PRESENT A CLEAN, NEAT APPEARANCE.

W.  FLAGS/PENNANTS.  THE UNION JACK, ENSIGN, AND ALL APPLICABLE

PENNANTS SHALL BE FLOWN - CLEAN, NOT WORN OR FRAYED.  HALYARDS SHALL

BE TAUT.  FLAGS/PENNANTS SHALL BE TWO-BLOCKED UNLESS AT HALF

MAST/AT-THE-DIP.

X.  TOPSIDE LIGHTS.  ALL REQUIRED LIGHTS SHALL BE IN WORKING ORDER

WITH APPLICABLE SAFETY COVERS IN PLACE, FREE FROM CRACKS.

Y.  COVERINGS FOR TOPSIDE EQUIPMENT.  TOPSIDE EQUIPMENT THAT HAS

PROTECTIVE COVERS SHALL HAVE THE COVERS INSTALLED.  COVERS SHALL BE

SECURELY FASTENED, TAUT, AND CLEAN.

Z.  PIER AND SURROUNDING AREAS.  PIER CLEANLINESS AND SMARTNESS ARE

AN EXTENSION OF TOPSIDE APPEARANCE.  EVERY PIER, AND ASSOCIATED AREA

OF RESPONSIBILITY, WILL MEET THE STANDARDS SET FORTH IN REFS I AND

J.  READ AND UNDERSTAND YOUR APPLICABLE SOPA INSTRUCTION.  EVERY

PIER SOPA WILL PROVIDE THE LEADERSHIP AND OVERSIGHT OF PIER

CLEANLINESS.  IN ADDITION TO THE PIER, PAY PARTICULAR ATTENTION TO

THE GUARDHOUSE, PARKING LOTS, AND COMMON AREAS ASSIGNED PER

APPLICABLE SOPA INSTRUCTION.  EACH SHIP ASSIGNED UNDER A PIER SOPA

SHALL FOLLOW SOPA DIRECTION AND SUPPORT THE SOPA IAW REFS I AND J.

4.  RADM THOMAS SENDS.//

BT

#0039